Scheduled Event Technical Requirements

Scenario: 	Scheduled Event – Option 1

Source: 		One or more IP Cameras

Connection:	RJ45 Ethernet on LAN to Linux Server

Video Codec:	H.264 or H.265

Resolutions        	720P, 1080P, or 4K

FPS                       	Usually 30, some options 60 FPS; Bit Rate 2 – 10 Mbps per stream

Protocol:	RTSP (maybe RTMP from some newer cameras)

Recording:	Continuous play (this create one or two very long files in the PIP, the streams are viewed on demand)

Live Tagging:	Analyst using iPad, laptop or Stream Deck

Requirement 1:	Save RTSP streaming files in a PIP on server for analysis 	Feature: Extract compressed video from RTSP, save as MP4 files on Linux Server

Requirement 2:	Save camera HDMI files in a PIP on server for analysis	Feature: Not possible with IP Cameras as video source

Requirement 3:    Coaches review plays, on demand, on iPad			Feature: Server converts RTSP stream to HSL or WebRTC, links with tagging data

Requirement 4:	Coaches review plays, on demand, on Win10 laptop		Feature: Server converts RTSP stream to HSL or WebRTC, links with tagging data

Requirement 5:	Stream video to 1 – 2 web browsers on Internet		Feature: Fast Internet connection, pass RTSP to small audience on web browsers

Requirement 6:	Stream video to dozens of fans or more, on the Internet	Feature: Convert RTSP stream to RTMP, send video stream to social media sites

Requirement 7:	Basic tagging using iPad or Stream Deck			Feature: Send tagging data live updates to server to provide time sync with video

Requirement 8: 	Add graphics, score, and narration to stream 		Feature: Not possible with IP Cameras as video source without special equipment

Concerns:	TBD

Scenario: 	Scheduled event – option 2

Source: 		One or more camcorders with streaming feature

Connection:	RJ45 Ethernet on LAN or Wi-Fi, and/or HDMI port capture to Linux Server

Video Codec:	H.264 or H.265

Resolutions        	720P, 1080P, (4K streaming not currently available from camcorders)

FPS                       	30, 60; Bit Rate 4 – 20 Mbps

Protocol:	RTSP or RTMP, HDMI

Recording:	Continuous play with occasional intermissions (this creates multiple longer files, no Internet streaming when not recording)

Live Tagging:	Analyst using iPad, laptop or Stream Deck

Requirement 1:	Save RTSP streaming files in a PIP on server for analysis	Feature: Extract compressed video, save as MP4 files on Linux Server

Requirement 2:	Save camera HDMI files in a PIP on server for analysis	Feature: Capture card to convert HDMI to files; must copy files to iPad for review

Requirement 3:    Coaches review plays, on demand, on iPad			Feature: Server converts RTSP stream to HSL or WebRTC, links with tagging data

Requirement 4:	Coaches review plays, on demand, on Win10 laptop		Feature: Server converts RTSP streams to required format, may also move mp4 files

Requirement 5:	Stream video to 1 – 2 web browsers on Internet		Feature: Fast Internet connection, pass RTSP to small audience on web browsers

Requirement 6:	Stream video to dozens of fans or more, on the Internet	Feature: Convert RTSP stream to RTMP, send video stream to social media sites

Requirement 7:	Basic tagging using iPad or Stream Deck			Feature: Send tagging data live updates to server to provide time sync with video

Requirement 8: 	Add graphics, score, and narration to stream 		Feature: Possible with Camcorder over HDMI as video source

Concerns:	Using both RSTP capture and HDMI video results in an incompatible mix of sources, some video can be streamed, some needs to be copied

Scenario: 	Scheduled event – option 3

Source: 		One or more camcorders with streaming feature

Connection:	RJ45 Ethernet on LAN or Wi-Fi, and/or HDMI port capture to Linux Server

Video Codec:	H.264 or H.265

Resolutions        	720P, 1080P, (4K streaming not currently available from camcorders)

FPS                       	30, 60; Bit Rate 4 – 20 Mbps

Protocol:	RTSP or RTMP, HDMI

Recording:	Record every play, but pause recording between plays (this creates many smaller, shorter files, no Internet streaming when not recording)

Live Tagging:	Analyst using iPad, laptop or Stream Deck

Requirement 1:	Save RTSP streaming files in a PIP on server for analysis	Feature: Extract compressed video, save large number of MP4 files on Linux Server

Feature: Merge the many small MP4 clips into one larger file for convenience

Requirement 2:	Save camera HDMI files in a PIP on server for analysis	Feature: Capture card to convert HDMI to files; must copy files to iPad for review

Requirement 3:    Coaches review plays, on demand, on iPad			Feature: Probably easiest to copy the smaller files to the iPad as needed

Requirement 4:	Coaches review plays, on demand, on Win10 laptop		Feature: Probably easiest to copy the smaller files to the laptop as needed

Requirement 5:	Stream video to 1 – 2 web browsers on Internet		Feature: Not very useful if only short segments are created, consider options

Requirement 6:	Stream video to dozens of fans or more, on the Internet	Feature: Not very useful if only short segments are created, consider options

Requirement 7:	Basic tagging using iPad or Stream Deck			Feature: Send tagging data live updates to server to provide time sync with video

Requirement 8: 	Add graphics, score, and narration to stream 		Feature: Not very useful if only short segments are created, consider options

Concerns:	If only short 10 – 20 second clips are available, may decide to only send highlight scenes to social media

Scenario: 	Scheduled event – option 4

Source: 		Multiple users with mobile devices such as iPad or Android phones

Connection:	Using Wi-Fi to send many short MP4 or MOV clips to the Linux Server

Video Codec:	H.264 or H.265

Resolutions        	720P, 1080P, 4K

FPS                       	30, 60; Bit Rate 5 – 10 Mbps

Protocol:	MP4 or MOV

Recording:	Record every play, but pause recording between plays (this creates many smaller, shorter files, no Internet streaming when not recording)

Live Tagging:	Analyst using iPad, laptop or Stream Deck

Requirement 1:	Save MP4 video files in a PIP on server for analysis		Feature: May merge many small MP4 clips into one larger file for convenience

Requirement 2:	Save camera HDMI files in a PIP on server for analysis	Feature: Not available on mobile devices

Requirement 3:    Coaches review plays, on demand, on iPad			Feature: Copy the smaller files to the iPad as needed

Requirement 4:	Coaches review plays, on demand, on Win10 laptop		Feature: Copy the smaller files to the laptop as needed

Requirement 5:	Stream video to 1 – 2 web browsers on Internet		Feature: Not practical for more than 1 viewer from a phone

Requirement 6:	Stream video to dozens of fans or more, on the Internet	Feature: Not possible with MP4 files unless streaming app is available

Requirement 7:	Basic tagging using iPad or Stream Deck			Feature: Send tagging data live updates to server to provide time sync with video

Requirement 8: 	Add graphics, score, and narration to stream 		Feature: Not available on mobile devices

Concerns:	This is a workable solution for smaller events, but with lower quality video recording from small lenses

Requirements Summary							Status

Convert H.264 RTSP streaming video to an .mp4 file.				Currently working

Convert H.265 RTSP streaming video to an .mp4 file.				May require some changes to support H.265

Convert RTSP streaming video to RTMP protocol in Linux server.		Some open source tools exist

Convert RTSP streaming video to WebRTC protocol in Linux server.		Some open source tools exist

Convert RTSP streaming video to HSL protocol in Linux server.			Some open source tools exist

Host a streaming server in a Docker container.				Some open source tools exist

Stream WebRTC or HSL from Linux server to iPad or Win10 laptop.		Some open source tools may exist

Copy mp4 video files from Linux server to iPad or Win10 laptop.		Some open source services exist

Stream RTSP video from IP camera to open Internet.				Not really practical for security and performance reasons

Stream RTSP video from Linux server to open Internet.			Still only supports a couple viewers without performance impact

Stream RTMP video to social media (i.e. YouTube).				Some open source tools exist, one to many streaming

Merge small mp4 files into larger clips for handling.				Some open source tools exist

Control IP cameras from Stream Deck or iPad.				Evaluate potential for device control

Control camcorder from Stream Deck or iPad.				Evaluate potential for device control

Control Linux Server workflow from Stream Deck or iPad.			Evaluate potential for device control

Linux server needs to perform multiple tasks concurrently.			Will be part of the microservices design

Scenario: 	Unscheduled event – option 5

Source: 		DSLR Camera or MicroSD card; iPhone or Android

Connection:	Use laptop or desktop to copy video clips to Pip

Video Codec:	H.264 or H.265

Resolutions        	720P, 1080P, 4K

FPS                       	30, 60, 120, 240; Bit Rate 10 – 40 Mbps

Protocol:	MP4 or MOV

Recording:	Record scenes as needed

Post Tagging:	Tagging after the event or scene was recorded

Requirement 1:	Save MP4 video files in a PIP on server for analysis		Feature: May merge many small MP4 clips into one larger file for convenience

Requirement 2:	Save camera HDMI files in a PIP on server for analysis	Feature: Not available on mobile devices

Requirement 3:    Coaches review plays, on demand, on iPad			Feature: Not applicable

Requirement 4:	Coaches review plays, on demand, on Win10 laptop		Feature: Not applicable

Requirement 5:	Stream video to 1 – 2 web browsers on Internet		Feature: Not applicable

Requirement 6:	Stream video to dozens of fans or more, on the Internet	Feature: Not applicable

Requirement 7:	Basic tagging using iPad or Stream Deck			Feature: Perform detailed tagging during analysis stage in Win10 application

Requirement 7: 	Add graphics, score, and narration to video 			Feature: Use tagging feature in Win10 application

Concerns:	This is my golf example.  Need to elaborate on the tagging and export video highlights to the cloud.